Fan with Motor

Code

//Small Fan

/****************************************************************

* As explained above, the amount of times you press the button should change the rotation speed of the fan.

* Pressing it once will cause it to rotate slowly,

* while pressing it three times will cause it to rotate quickly, and pressing it four times will cause it to stop.

****************************************************************/

// constants won't change. They're used here to

// set pin numbers:

const int buttonPin = 2;    // the number of the pushbutton pin

const int ledPin = 13;      // the number of the LED pin

const int motorIn1 = 9;

const int motorIn2 = 10;

int stat = 0;

#define rank1 150

#define rank2 200

#define rank3 250

// Variables will change:

int buttonState;             // the current reading from the input pin

int lastButtonState = LOW;   // the previous reading from the input pin

// the following variables are long's because the time, measured in miliseconds,

// will quickly become a bigger number than can be stored in an int.

long lastDebounceTime = 0;  // the last time the output pin was toggled

long debounceDelay = 50;    // the debounce time; increase if the output flickers

/******************************************************************************/

void setup()

{

//set theled,motors as OUTPUT,button as INPUT

pinMode(buttonPin, INPUT);

pinMode(ledPin, OUTPUT);

pinMode(motorIn1,OUTPUT);

pinMode(motorIn2,OUTPUT);

Serial.begin(9600);

}

void loop() {

// read the state of the switch into a local variable:

int reading = digitalRead(buttonPin);

if (reading != lastButtonState)// If the button state is different from last time

{

lastDebounceTime = millis();// reset the debouncing timer

}

if ((millis() - lastDebounceTime) > debounceDelay)

{

if (reading != buttonState)

{

buttonState = reading; // Store the state of button in buttonState

// only toggle the LED if the new button state is HIGH

if (buttonState == HIGH)

{

digitalWrite(ledPin, HIGH); //turn on the LED

stat = stat + 1;

if(stat >= 4)// When stat>=4, set it as 0.

{

stat = 0;

}

}

else

digitalWrite(ledPin, LOW);

}

}

switch(stat)

{

case 1:

clockwise(rank1);// When stat=1, set the rotate speed of the motor as rank1=150

break;

case 2:

clockwise(rank2);// When stat=2, set the rotate speed of the motor as rank1=200

break;

case 3:

clockwise(rank3);// When stat=3, set the rotate speed of the motor as rank1=250

break;

default:

clockwise(0);// else, set the rotate speed of the motor as rank1=150

}

// save the reading.  Next time through the loop,

// it'll be the lastButtonState:

lastButtonState = reading;

}

/***********************************************************/

void clockwise(int Speed)//

{

analogWrite(motorIn1,0);

analogWrite(motorIn2,Speed);

}

/***********************************************************/